Supplemental Method S2

Experimental assessment of droplet size

We have measured droplet polydispersity by using transparent Sapphire Chips, which can be imaged in bright light using a standard microscope.

Droplet crystals were generated in the 12 chambers of 3 different chips using the standard partitioning protocol. The chips were then recovered for imaging prior to performing PCR amplification.

We used a Nikon Eclipse Ti-E inverted microscope with a 10X objective (1.08 microns / pixel resolution) to image the droplet crystal. For each chamber, we took images of the droplet crystal in 10 to 15 different locations in the crystal, imaging 1500 to 3000 droplets per chamber with a 1.08 microns resolution, amounting to a total of 34,000 individual droplets imaged.

The droplet diameter measurement was performed using an automated ImageJ script. We obtained a standard deviation of 2.41 pixels for an average droplet diameter of 86 pixels, translating to 2.8% diameter polydispersity (STD/Average). This therefore yields an average droplet volume of 0,435 nL with a 0,034 nL STD and a 8.4% volume polydispersity (STD/Average).